# ETINCELL® | Single blade louvre – mounted on carriers U 1000 H

Single blade open ceiling consisting of a substructure [U 1000 H] and prefabricated panel elements.

The sendzimir galvanized clip-in carrier U 1000  H is suspended rigidly from the bare ceiling as cross bracing [primary profile] with nonius suspension elements [lower part: U 1004, upper part: U 1300, connector pin U 1372] or with quick fixing hangers [suspension hanger: U 1051, suspension wire: U 035] using officially approved dowels.

The primary profiles must be connected to one another at the longitudinal joints using U 1002 longitudinal connectors.

The primary profiles are joined with the walls using the wall bracket U 1010.

The spacing of the primary profiles depends on the requirements of DIN EN 13964 and the statics of the system and must be proven and established by the contractor.

Ensure horizontal and flush alignment during installation.

The primary profiles U 1000 H are connected to the secondary profiles U 1000 H using U 1001 H cross connectors.

The secondary profiles are connected to one another at the longitudinal joint using connector U 1002.

The U 1010 wall bracket is used for joining the secondary profiles to the wall.

The spacing of the secondary profiles must be adjusted precisely to the panel size.

Ensure horizontal and flush alignment during installation.

The blades of the ETINCELL® single blade louvre are perforated. The panels are mounted at the module spacing and consist of square cells. The individual panel elements are hung into the carrier concealed or semi-concealed. The individual panels are self-adjusting.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – ETINCELL® single blade open ceiling Panel size [LxW]:  600x600mm  625x625mm  1200x600mm  1250x625mm Material thickness: 0.6mm Cell dimension: ............... mm [25-60mm] Blade height: ............... mm [25-50mm] Colour:  White  Black  Natural anodized  RAL 9006  RAL 7012 Material: Aluminium [pre-coated on both sides, edges not coated] Unit price: …............ €/m² Total price: …............ € |
| --- | --- |
| Pos. m² | Carrier [U 1000 H] Material: Sendzimir galvanized steel Length: 4000mm Unit price: …............ €/m² Total price: …............ € |
| Pos. pcs | Carrier connector [U 1002] Material: Steel Unit price: ............… €/pc Total price: ............… € |
| Pos. pcs | Carrier cross connector [U 1001 H] Material: Steel Unit price: ............… €/pc Total price: ............… € |
| Pos. pcs | Wall bracket [U 1010] Material: Steel Unit price: ............… €/pc Total price: ............… € |