# FF025  |  Lay-in system without shadow gap

| Pos............................ ..............................m² Pos............................ Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System FF025. The metal panels are cut to fit in the border profile [see separate position] on one side to compensate for tolerance and slid in. On the other side, the panel is laid form-fitting and tension-free in the profile. Easy demounting without tools is ensured. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s=approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm €/m² Start-up costs depending on format, training and call-up € Delivery and installation of durlum F 025 profiles of aluminium. The profiles are to be mounted on the one side of the corridor with the insertion lip horizontal for the insertion of the cut-off metal panels and vertically on the opposite side of the corridor wall so that the long-field panels can be laid on them. The profiles are to be attached completely horizontal. Only officially approved dowel plugs may be used. The spacing of the attachment points is according to DIN 18168, DIN EN 13964 and the loads of the system and to be checked by the contractor. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. Material thickness: 1.5 mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Profile form: According to durlum F 025 without shadow gap 25/25 mm €/running m |
| --- | --- |