# FS1.12  |  Clip-in system

| Pos............................ ..............................m2 Pos............................ Pos............................ ..................running m Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System FS1.12. The metal panels are positively latched into a special substructure [separate position] by means of clips. To cover the clip carrier rail by the metal panels, they are given a special bend on the ends. With a demounting tool, the metal panels can be pulled out of the clip construction. Free access to the ceiling space is ensured. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or brushed stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s=approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm [Panel weight. 4.0 kg/panel] €/m2 Start-up costs depending on format, training and call-up € Delivery and installation of a System FS1.12 substructure consisting of sendzimir galvanised U 1000 clip-in profiles that are suspended pressure-rigid from the bare ceiling by means of nonius lower and upper parts or with WK 1050 angle brackets and S 2545 slide pieces on the parallel running corridor walls with officially approved dowels plugs. The profiles are to be attached completely horizontal. The spacing of the attachment points or the suspension hangers is according to DIN 18168, DIN EN 13964 and the loads of the system and is to be checked by the contractor. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. All parts are made of galvanised steel. €/running m Delivery and installation of durlum AW 2560 cover angles for covering the gap between the metal panels and the corridor walls. The cover angles are to be fitted on site to the gap width and attached to the corridor walls above the metal panels with dowel plugs. Material: Optionally galvanised steel or brushed stainless steel Material thickness: According to strength requirements Surface finish: D206-700 white similar to RAL 9016, optional colour RAL ............... €/running m |
| --- | --- |