# FS4.2  |  Lay-on/hook-on system

| Pos............................ ..............................m² Pos............................ Pos............................ ..................running m Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System FS4.2. The metal panels are hung form-fitting und tension-free by means of a special substructure [see separate position]. Demounting without tools is ensured. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s=approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm €/m² Start-up costs depending on format, training and call-up € Delivery and installation of a durlum System FS4.2 substructure consisting of U 1005 A hook-on carrier Z-profiles which are mounted lengthwise to the corridor. The hook-on carrier Z-profiles are hung either with L-shaped primary carrier with regular perforations mounted laterally to the corridor as a lateral grid and nonius lower and upper parts or without primary carriers with threaded rods or nonius lower and upper parts from the bare ceiling or with U 1042 wall brackets to the corridor walls in a pressure-rigid manner with officially approved dowel plugs. The spacing of the suspension points is according to DIN 18168 and DIN EN 13964 as well as the loads of the system and is to be determined and checked by the contractor. Care is to be taken to ensure horizontal and flush mounting. Threaded connections are to be secured against loosening. This system can accommodate tolerances of 10-40mm to the corridor walls. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. All parts are made of galvanised steel. €/running m Delivery and installation of durlum AW 2560 cover angles for covering the gap between the metal panels and the corridor walls. The cover angle are to be fitted on site to the gap width and attached to the corridor walls above the metal panels with dowel plugs. This system can accommodate tolerances of 20-40mm to the corridor walls. Material: Optionally galvanised steel or brushed stainless steel Material thickness: According to strength requirements Surface finish: D206-700 white similar to RAL 9016, optional colour RAL ............... €/running m |
| --- | --- |