# FS4.6 BR  |  Lay-on/pin locking system, hinged/movable

| Pos............................ ..............................m² Pos............................ Pos............................ ..................running m Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System FS4.6 BR. The metal panels are hung by means of a special substructure [see separate position] form-fitting und tension-free with spring-mounted form pins attached on the ends of the metal panels The folding down and demounting takes place with simple tools [e.g. screwdriver]. The folded down metal panels can be effortlessly slid together to form a package. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s=approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm €/m² Start-up costs depending on format, training and call-up € Delivery and installation of a durlum System FS4.6 BR substructure consisting of U 1091 perimeter hook-on profiles which are mounted lengthwise to the corridor. The perimeter hook-on profiles are mounted either with U 1042 wall brackets to the corridor walls or with nonius lower and upper parts with officially approved dowel plugs in a pressure-rigid manner to the corridor walls. As an alternative, suspended with threaded rods or with U 1040 primary carriers mounted laterally to the corridor and nonius lower and upper parts and officially approved dowel plugs in a pressure-rigid manner to the bare ceiling Screw connections are to be secured against loosening. The spacing of the attachment points is according to DIN 18168 and DIN EN 13964 as well as the loads of the system and is to be determined and checked by the contractor. Care is to be taken to ensure horizontal and flush mounting as well as parallel alignment and sway-free mounting of the hook-on U-profiles. The system can accommodate width variations between the walls of 0-40 mm. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. All parts are made of galvanised steel. €/running m Delivery and installation of durlum AW 2560 cover angles to cover the gap between the metal panels and the corridor walls and mount the support profiles U 1091. The cover angles are to be fitted on site to the gap width and attached to the corridor walls above the metal panels with dowel plugs. Material: Optionally galvanised steel or brushed stainless steel Material thickness: According to strength requirements Surface finish: D206-700 white similar to RAL 9016, optional colour RAL ............... €/running m |
| --- | --- |