# PYTHA® | Multidirectional louvre – mounted with integrated carrier

Multidirectional louvre consisting of a substructure with integrated carrier and prefabricated panel elements.

The panel element is suspended rigidly from the mounting level [e.g. bare ceiling] with a G9 suspension hanger by means of M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964]. The distance between the carriers is 1200mm.

The panel elements are mounted at the module spacing and consist of triangular cells with U-shaped sharp-edged blades. The parallelogram-shaped panel element consists of different longitudinal and cross blades. The blades of the short and long sides are connected form-fitting to the integrated carrier by means of the formed hooks. The individual panels are self-adjusting and can be removed individually. The integrated transverse ensures the correct alignment of the panel elements.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – PYTHA® multidirectional open cell ceiling Panel size: 1385.64x600mm [shape of a parallelogram] Blade width/height: 9/40mm Step:  86mm  100mm  120mm  150mm Other steps on request Colour: ............... [Other colours on request] Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. rm | L-shaped perimeter trims Height: 40mm Width: 20mm Length: 2000mm Material: Aluminium, pre-coated Colour:  White  Black  RAL 9006  Natural anodized Unit price: ............… €/rm Total price: ............… € |