# QUADRA® | Single blade louvre – mounted on T15/T24 carrier

Single blade open ceiling consisting of a substructure [T-shaped rail] and prefabricated panel elements.

The T-shaped rail is suspended rigidly from the mounting level [e.g. bare ceiling] via a suspension hanger for threaded rod with M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964]. The distance between the T-profiles depends on the specified module dimension – according to the panel size length.

The panel elements are mounted at the module spacing. The individual panels are self-adjusting. The individual panel elements are placed on the T-profiles [visible construction] or hung into them [semi-concealed construction].

Typically, the edge connection is carried out by means of a shadow gap.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – QUADRA® single blade open ceiling Panel size [LxW]:  600x600mm  625x625mm  1200x600mm  1250x625mm Material thickness: 0.6mm Cell dimension: ............... mm [17-100mm] Blade height: ............... mm [19-62mm] Colour:  White  Black  Natural anodized  RAL 9006  RAL 7012 Material: Aluminium [pre-coated on both sides, edges not coated] Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. m² | T-shaped rail [T15/T24] Colour:  White Other colours on request Unit price: ............… €/m² Total price: ............… € |