# FS4.4 AS RHOMBOS  |  Attachment/hook system, adjustable in height/movable

| Pos............................ ..............................m² Pos............................ Pos............................ .............................running metre Pos............................ .............................running metre | Delivery and installation of durlum expanded metal panels of the FS4.4 AS RHOMBOS V1 system. A frame is butt welded onto the cut-to-size expanded metal panels. By this, precisely fitting parts are produced that are hung into the holders located at the short sides of the expanded metal panels in a positive-locking and tension-free way by means of a special substructure [see separate position]. To lower or move the panels, they must be removed from the holders, lowered and hooked into the hooks pointing to the outside. Then they can be moved. Tool-free dismounting is guaranteed. The hook-in and unhooking system operates in opposite directions to prevent panels from falling out during dismounting Tolerances and quality requirements apply according to TAIM, DIN EN 13964 and durlum standard. Material: electrolytically galvanised steel Variant: V1 (surface-welded frame) Mesh M260_1.5 Mesh length: 28 mm Mesh width: 12 mm Web width: 2.0 mm Web thickness: 1.5 mm Exp. metal thickness: approx. 3.6 mm Surface: Powder-coated similar to RAL9006-079 smooth and shiny (alternative RAL......) Layer thickness: approx. 60 µm Sound absorption: with a special durlum acoustic fleece and support Colour: black Dimension: Length ............... mm Width ............... mm € / m2 Set-up costs depending on format, design and call-off € Delivery and installation of a durlum FS4.4 AS substructure consisting of U-shaped hook-on profiles U 1087 which are to be mounted in longitudinal direction of the corridor system. The U-shaped hook-on profiles are mounted with wall brackets U 1042 to the corridor walls or with vernier scale upper and lower parts to the bare ceiling or the corridor walls to ensure a rigid suspension. As an alternative, the U-shaped hook-on profiles can be mounted to the the bare ceiling by means of perforated L-shaped primary carriers U 1040 mounted transversely to the corridor as cross-bracing and vernier scale upper and lower parts with officially approved dowel to ensure a rigid substructure. All screw connections must be secured against loosening, The spacing of the suspension points depends on the requirements of DIN EN 13964 and the statics of the system and must be proven and established by the contractor. Ensure horizontal, flush, parallel and swing-free mounting of the U-shaped hook-on profile. The system is able to compensate spacings of 20-40 mm to the corridor walls. Only constructions may be used that have been approved by the manufacturer of the expanded metal panels. All parts are made of galvanised sheet steel. €/running metre Delivery and installation of durlum perimeter trims AW 2560 to cover the joints between the rectangular metal panel and the corridor walls and for the suspension of the U 1087. The perimeter trims must be adapted on site to the joint widths and attached with dowels above the rectangular metal panels on the corridor walls. Material: galvanised sheet steel or brushed stainless steel Material thickness: according to static requirements Surface: Powder-coated similar to RAL9006-079 smooth and shiny (alternative RAL......) €/running metre |
| --- | --- |