# S10 H RHOMBOS | Double hook system

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ .............................running metre | Delivery and installation of durlum expanded metal panels of the S10 H RHOMBOS V1 system. A frame is butt welded onto the cut-to-size expanded metal panels. By this, precisely fitting parts are produced which are hung in a positive-locking and tension-free way into the suspension system via a special substructure [see separate position] using special recesses in the long sides of the expanded metal panels. Tool-free dismounting is guaranteed. Tolerances and quality requirement according to the durlum standard. Material: electrolytically galvanised steel Variant: V1 (surface-welded frame) Mesh M260_1.5 Mesh length: 28 mm Mesh width: 12 mm Web width: 2.0 mm Web thickness: 1.5 mm Exp. metal thickness: approx. 3.6 mm Surface: Powder-coated similar to RAL9006-079 smooth and shiny (alternative RAL......) Layer thickness: approx. 60 µm Sound absorption: with a special durlum acoustic fleece and support Colour: black Dimension: Length ............... mm Width ............... mm € / m2 Set-up costs depending on format, design and call-up. € Delivery and installation of a durlum S10 H RHOMBOS substructure consisting of a perforated L-shaped primary carrier U1040 as cross bracing (primary profiles) which is suspended from the bare ceiling with the help of vernier scale lower and upper parts or with threaded rods using officially approved dowels in order to ensure a rigid suspension. The primary profiles must be connected [screwed] to one another using longitudinal connectors U 1041 at the longitudinal joint. The distance between the primary profiles has to be defined according to DIN EN 13964 and the mounting instructions and must be proven and established by the contractor. Double hook-on profiles U 1093 are fastened to the primary profiles as longitudinal profiles (secondary profiles) by means of cross connectors U 1093-V and FIXCLIP. The primary and secondary profiles are mounted to the walls using wall brackets U 1042. The spacing of the secondary profiles is to be adjusted precisely to the length of the expanded metal panels and the recesses to ensure tension-free suspension of the panels in the system. Ensure horizontal, flush and swing-free alignment during installation. Only constructions may be used that have been approved by the manufacturer of the expanded metal panels. All parts are made of galvanised steel sheet or aluminium. € / m2 Delivery and installation of durlum wall connection profiles made of aluminium. Material thickness: 1.5 mm Surface: Powder-coated similar to RAL9006-079 smooth and shiny (alternative RAL......) Profile form: according to durlum perimeter trim without shadow gap 25/25 mm or according to durlum stepped perimeter trim 25/20/20/25 mm € / running metre |
| --- | --- |