# S4 RHOMBOS | Hook-on system

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..........running metre | Delivery and installation of durlum expanded metal panels of the S4 RHOMBOS V1 system. A frame is butt welded onto the cut-to-size expanded metal panels. By this, precisely fitting parts are produced that are hung into a special substructure [separate position] in a positive-locking and tension-free way. Tool-free dismounting is guaranteed. Tolerances and quality requirement according to the durlum standard. Material: electrolytically galvanised steel Variant: V1 [surface-welded frame ] Mesh M260_1.5 Mesh length: 28 mm Mesh width: 12 mm Web width: 2.0 mm Web thickness: 1.5 mm Exp. metal thickness: approx. 3.6 mm Surface finish: Powder-coated similar to RAL9006-079 smooth and shiny [alternative RAL...... ] Layer thickness: approx. 60 µm Sound absorption: with a special durlum acoustic fleece and support Colour: black Dimension: Length ............... mm Width ............... mm € / m2 Set-up costs depending on format, design and call-up. € Delivery and installation of a durlum S4 RHOMOBS substructure consisting of a perforated L-shaped primary carrier U 1040 as cross-bracing [primary profile ] which is suspended from the bare ceiling with the help of vernier scale upper and lower parts or with threaded rods using officially approved dowels in order to ensure a rigid suspension. The primary profiles must be connected [screwed] to one another using longitudinal connectors U 1041 at the longitudinal joint. The distance between the primary profiles has to be defined according to DIN EN 13964 and the installation manual and must be proven and established by the contractor. The hook-on carriers U 1005 A are screwed as longitudinal profiles [secondary profiles ] to the primary profiles using screws M6 [secured against loosening]. The longitudinal connection of the secondary profiles is performed using longitudinal connectors U 401. The primary and secondary profiles are mounted to the walls using wall brackets U 1042. The spacing of the secondary profiles is exactly matched to the spacing of the hang-in side of the expanded metal panels so that the panels lie tension free in the system. Ensure horizontal and flush alignment during installation. Only constructions may be used that have been approved by the manufacturer of the expanded metal panels. All parts are of galvanised steel sheet metal. Suspension height: ........... mm. € / m2 Delivery and installation of durlum wall connection profiles made of aluminium. Material thickness: 1.5 mm Surface finish: Powder-coated similar to RAL9006-079 smooth and shiny [alternative RAL...... ] Profile form: according to durlum perimeter trim without shadow gap 25/25 mm or according to durlum stepped perimeter trim 25/20/20/25 mm € / running metre |
| --- | --- |