# dur-SOLO® RHOMBOS type 2  |  Consisting of several parts

| Pos............................ ..............................pcs raft ceiling elements Pos............................ Pos............................ ..............................pcs raft ceiling elements | Delivery and installation of dur-SOLO® RHOMBOS – type 2 V1 A frame is butt welded onto the cut-to-size expanded metal panels. By this, precisely fitting parts are produced. The raft ceiling elements are screwed to one another and inserted into a special substructure [see separate position] in a positive-locking and tension-free way. Tool-free dismounting is guaranteed. Tolerances and quality requirements apply according to TAIM, DIN EN 13964 and durlum standard. Material: electrolytically galvanised steel Variant: V1 (surface-welded frame) Mesh M260_1.5 Mesh length: 28 mm Mesh width: 12 mm Web width: 2.0 mm Web thickness: 1.5 mm Exp. metal thickness: approx. 3.6 mm Surface: Powder-coated similar to RAL9006-079 smooth and shiny (alternative RAL......) Layer thickness: approx. 60 µm Sound absorption: with a special durlum acoustic fleece and support Colour: black Dimension: Length ............... mm Width ............... mm € / m2 Set-up costs depending on format, design and call-off € Delivery and installation of a durlum substructure consisting of at least three transverse braces which are suspended from the ceiling by means of two threaded rods for each brace. Suspension height: ........... mm Only construction parts may be used that have been approved by the manufacturer of the raft ceiling. All parts are made of galvanised sheet steel. €/raft ceiling element |
| --- | --- |