# S1.12 KD  |  Clip-in system, hinged/movable

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System S1.12 KD. Two dur-stirrups are fitted to every panel, one at each short side. The metal panels are attached to a special substructure [separate position]. The panels are held by friction in special clamping strips. Using an opening tool, the metal panels can be pulled out of the retaining structure, hinged on the dur-stirrups, and pushed together so as to provide free access to the hollow space. Tolerance and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s= approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm [Panel weight 4.0 kg/panel] €/m2 Start-up costs depending on format, training and call-up. € Delivery and installation of a System S1.12 KD substructure consisting of sendzimir galvanised U 1000 clip-in profiles that are suspended as a gird construction by means of U 1001 H carrier rail cross connectors. The spacing of the gird clip-in profiles is to be laid-out according to DIN 18168, DIN EN 13964 and the loads of the systems and is to be checked by the contractor. The profiles are suspended pressure-rigid by means of nonius adjustable carriers. Suspension height: ............... mm. Care is to be taken to ensure horizontal and flush alignment. Only officially approved dowels plugs many be used. The wall connection of the clip-in profiles are made with U 1010 wall bracket parts. The spacing of the main clip-in profiles is to be matched exactly with the spacing of the two clip-in sides of the metal panels. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. €/m2 Delivery and installation of durlum wall connection profiles of aluminium. Material thickness: 1.5 mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Profile form: According to durlum F 025 perimeter trim without shadow gap 25/25 mm or durlum F 2025 with stepped perimeter trim 25/20/20/25 mm €/running m |
| --- | --- |