# S1.12  |  Clip-in system, removable

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System S1.12. The metal panels are positively latched into position in a special substructure [separate position] by means of clips. With a demounting tool, the metal panels can be pulled out of the clip construction and removed. Free access to the ceiling space is ensured. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s= approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm [Panel weight 4.0 kg/panel] €/m2 Start-up costs depending on format, training and call-up € Delivery and installation of a System S1.12 substructure consisting of sendzimir galvanised U 1000 clip-in profiles that are suspended as a gird construction by means of U 1001 H carrier rail cross connectors. The spacing of the gird clip-in profiles is to be laid-out according to DIN 18168, DIN EN 13964 and the loads of the systems and is to be checked by the contractor. The profiles are suspended pressure-rigid by means of nonius adjustable carriers. Suspension height: ............... mm. Care is to be taken to ensure horizontal and flush alignment. Only officially approved dowels plugs many be used. The wall connection of the clip-in profiles are made with U 1010 wall bracket parts. The spacing of the main clip-in profiles is to be matched exactly to the spacing of the two clip-in sides of the metal panels. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. €/m2 Delivery and installation of durlum wall connection profiles of aluminium. Material thickness: 1.5mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Profile form: According to durlum F 025 perimeter trim without shadow gap 25/25 mm or durlum F 2025 with stepped perimeter trim 25/20/20/25 mm €/running m |
| --- | --- |