# S10 H  |  Double-hook system with panels hooked on the long side

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System S10 H. The metal panels are hooked form-fitting and tension-free into a special substructure [see separate position] with special latches attached to the long side of the metal panels. Demounting is preformed without tools. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel, etc. Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s= approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm €/m2 Start-up costs depending on format, training and call-up € Delivery and installation of a System S10 H substructure consisting of U 1040 form punched grid angles as lateral grid which are suspended from the bare ceiling with nonius adjustable lower and upper parts or threaded rods using official approved dowel plugs. The grid angles are to be connected together at the ends with U 1041 longitudinal connectors [threaded fasteners]. The spacing of the grid angles is according to the requirements of DIN 18168, DIN EN 13964 as well as the loads of the system and is to be determined and checked by the contractor. On the grid angles, U 1093 double-hook profiles as longitudinal profiles are attached on alternate sides with M6 bolts [secured against loosening]. The connection of the grid angles as well as the T-hook profiles to the walls is made with U 1042 wall brackets. The spacing of the double hook profiles is to be matched exactly to the length of the metal panels or the hook-in latch so that the panels rest tension-free in the system. Care is to be taken to ensure horizontal and flush alignment as well as sway-free mounting. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. All parts are made of galvanised steel or aluminium. €/m2 Delivery and installation of durlum wall connection profiles of aluminium. Material thickness: 1.5 mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Profile form: According to durlum F 025 perimeter trim without shadow gap 25/25 mm or durlum F 2025 with stepped perimeter trim 25/20/20/25 mm €/running m |
| --- | --- |