# S4  |  Hook-on system

| Pos............................ ..............................m2 Pos [Variant]............. ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System S4. The metal panels are suspended form-fitting and tension-free by means of a special substructure [see separate position]. Demounting is preformed without tools. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s= approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm €/m2 Variant: delivery and installation of durlum System Type TAIFUN rectangular metal panels. The rectangular panels are attached to a special substructure [see separate position] which provides positive, stress-free retention. Ball resistance is assured by a special ball protection element which is fitted between the two longer sides of neighbouring panels and is fixed to the Z-support profile. Panels can be dismounted individually. A joint width of 6mm is required on the long sides. Panels are simple to dismount. Ball resistance is demonstrated by a test certificate to DIN 18032 Part 3 and DIN EN 13964. Tolerances and quality requirements according to TAIM and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: 1 mm or as determined by structural considerations Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece as= approx. 0.73, optionally in black or white Dimensions: Length ............... mm [max. 1600 mm] Width ............... mm [max. 300 mm] €/m2 Start-up costs depending on format, training and call-up € Delivery and installation of a System S4 substructure consisting of U 1040 form punched angles as a lateral grid which is suspended pressure-rigid from the bare ceiling with nonius adjustable lower and upper parts or with threaded rods using official approved dowel plugs. The angles are to be connected together at the ends by means of U 1041 longitudinal connectors [screw fasteners]. The spacing of the grid angles is according to the requirements of DIN 18168 and DIN EN 13964 as well as the loads of the system and are to be determined and checked by the contractor. On the grid angles, U 1005 A Z-shaped carrier profiles are attached as longitudinal profile with M6 bolts [secured against loosening]. The longitudinal connection of the Z-shaped carrier profiles are made by means of U 401 profile connectors. The attachment of the grid angles to the walls as well as the carrier profiles is by means of U 1042 wall brackets. The spacing of the Z-shaped carries profiles is to be matched exactly to the spacing of the hook-in sides of the metal panels so that panels rest tension-free in the system. Care is to be taken to ensure horizontal and flush alignment. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. All parts are made of galvanised steel. €/m2 Delivery and installation of durlum wall connection profiles of aluminium. Material thickness: 1.5 mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Profile form: According to durlum F 025 perimeter trim without shadow gap 25/25 mm or durlum F 2025 with stepped perimeter trim 25/20/20/25 mm €/running m |
| --- | --- |