# S5.4 K-BR  |  C-channel parallel grid system, hinged with pivot and lock

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System S5.4 K-BR. The metal panels are latched form-fitting and tension-free in a special substructure [see separate position]. In the metal panels, spring mounted pin as well as latches are inserted and attached. For demounting, the latch is pressed back with a screwdriver and the metal panel is swung down. Tolerances and quality requirements according to TAIM; DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s= approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm €/m2 Start-up costs depending on format, training and call-up € Delivery and installation of a durlum System S5.4 K-BR substructure consisting of form punched U1040 grid angles as a lateral grid which are suspended from the bare ceiling with nonius adjustable lower and upper parts or threaded rods using official approved dowel plugs. Suspension height: ............... mm. The grid angles are to be connected at the ends by means of U 1041 longitudinal connectors [screw fasteners]. The spacing of the grid angles is according to the requirements of DIN 18168, DIN EN 13964 as well as the loads of the system and are to be determined and checked by the contractor. The attachment of the grid angles to the walls is with U 1042 wall brackets. On the grid angles, C 2000 C-channels as longitudinal profiles are attached by means of C 2007 channel profile hangers with M6 bolts [secured against loosening]. As an alternative, the C-channels can be suspended with the C 2003 junction hangers. On the longitudinal sides of the C-channels, special latches that are matched to the dimensions of the metal panels are provided for attachment of the metal panels. The longitudinal connection of the C-channels is made with C 2004 longitudinal connectors. The attachment to the walls is made with C 2009 wall brackets. The spacing of the C-channels is to be matched exact to the length of the metal panels so that the panels rest tension-free in the system. Care is to be taken to ensure horizontal and flush alignment. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels All parts are made of galvanised steel. C-channel surface finish: D206-700 white powder coated similar to RAL 9016, optional colour according to RAL ............... Alternative materials for the C-channels: Aluminium or stainless steel Coating thickness: Approx. 70 µm €/m2 Delivery and installation of durlum wall connection profiles of aluminium. Material thickness: 1.5 mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour according to RAL ............... Profile form: According to durlum F 025 perimeter trim without shadow gap 25/25 mm or durlum F 2025 with stepped perimeter trim 25/20/20/25 mm €/running m |
| --- | --- |