# S5.2-100  |  Junction-point C-channel grid system

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System S5.2-100. The metal panels are placed form-fitting and tension-free on a special substructure [see separate position]. Removal and reinsertion is preformed without tools. The metal panels can be flush mounted to the front or rear. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s= approx. 0.73, optionally in black or white Dimensions: Length ............... mm Module system Width ............... mm Module system €/m2 Start-up costs depending on format, training and call-up € Delivery and installation of a System S5.2-100 substructure consisting of CK 2003 bent cross boxes provided with hangers which are suspended pressure-rigid at specified distances [axis dimension] according to DIN 18168 and DIN EN 13964 from the bare ceiling with threaded rods or nonius adjustable lower and upper parts using official approved dowel plugs. Suspension height: ............... mm. The loads of the system are to be determined and checked by the contractor. Between the junction points, lateral and longitudinal running, exactly to length fabricated C-channels are laid form-fitting into the junction points. The attachment to the walls is made with C 2009 wall brackets. Between the C-channels, the exactly fabricated metal panels are to be laid tension-free into the system. Care is to be taken to ensure horizontal and flush alignment. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. All parts are made of galvanised steel. C-channel surface finish: D206-700 white powder coated similar to RAL 9016, optional colour according to RAL ............... Coating thickness: Approx. 70 µm C-channel material: Galvanised steel, optionally aluminium or stainless steel €/m2 Delivery and installation of durlum wall connection profiles of aluminium. Material thickness: 1.5 mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Profile form: According to durlum F 025 perimeter trim without shadow gap 25/25 mm or durlum F 2025 with stepped perimeter trim 25/20/20/25 mm €/running m |
| --- | --- |