# S7 KS  |  Rail channel system, hinged/movable

| Pos............................ ..............................m2 Pos............................ Pos............................ ..............................m2 Pos............................ ..................running m | Delivery and installation of durlum metal panels of the System S7 KS. The metal panels are hung form-fitting and tension-free by means of a special substructure [see separate position] with bent-up hooks on the ends of the metal panels into the rail channel profiles. To swing down, the panels are unhooked, lowered and then hung with the attached hooks on the ends and swung down. The swung down metal panels can be easily slid together to bundles. Demounting is preformed without tools. Tolerances and quality requirements according to TAIM, DIN EN 13964 and durlum Standards. Material: Optionally galvanised steel or aluminium/stainless steel Thickness: According to strength requirements Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm Perforations: durlum Type RG-L15 [2.5/5.5] Sound absorption: By means of a special durlum acoustic fleece s= approx. 0.73, optionally in black or white Dimensions: Length ............... mm Width ............... mm €/m2 Start-up costs depending on format, training and call-up € Delivery and installation of a System S7 KS substructure consisting of an aluminium rail channel profile which is suspended pressure-rigid and sway-free from the bare ceiling with nonius adjustable lower and upper parts using official approved dowel plugs. Suspension height: ............... mm. The longitudinal connection of the rail channel profiles are made with rail channel longitudinal connectors. The attachment to the walls is made with U 1042 wall brackets and threaded plates with bolts. The spacing of the rail channel profiles is to be matched exactly to the length of the ceiling panels so that the panels hang tension-free in the system. Care is to be taken to ensure horizontal and flush alignment. It is only permissible to use structure components that have been approved by the manufacturer of the metal panels. All parts are made of galvanised steel or aluminium. Surface finish of the rail channel profiles: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Coating thickness: Approx. 70 µm €/m2 Delivery and installation of durlum wall connection profiles of aluminium. Material thickness: 1.5 mm Surface finish: D206-700 white powder coated similar to RAL 9016, optional colour RAL ............... Profile form: According to durlum F 025 perimeter trim without shadow gap 25/25 mm or durlum F 2025 with stepped perimeter trim 25/20/20/25 mm €/running m |
| --- | --- |