# STARLAM® | Linear louvre – mounted with integrated carrier

Linear blade louvre consisting of a substructure with integrated carrier and prefabricated panel elements.

The panel element is suspended rigidly from the mounting level [e.g. bare ceiling] with a G4 suspension hanger by means of M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964].

The distance between the carriers depends on the specified module dimension – according to the element size length.

The panel elements are mounted at the module spacing and consist of cells with U-shaped, sharp-edged blades running in parallel. The blades are connected to a panel by the transverses. These transverses are connected form-fitting to the integrated carrier by means of the formed hooks. The individual panels are self-adjusting. The panels are hung into the carriers.

Typically, the edge connection is carried out by means of an L-shaped perimeter trim.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – STARLAM® linear blade open cell ceiling Panels:  1200x600mm  1250x625mm Blade:  STARLAM® 4: 4/42mm [blade width/blade height]  STARLAM® 9: 9/40mm [blade width/blade height]  STARLAM® 15: 15/37mm [blade width/blade height] Blade distance/module: ............... mm [30-75mm] Other distances on request Blade colour:  White  RAL 9006  Black Other colours on request Traverse:  STARLAM® 4: 15/37mm [blade width/blade height]  STARLAM® 9: 15//37mm [blade width/blade height]  STARLAM® 15: 15/37mm [blade width/blade height] Traverse distance: STARLAM® 4:  200mm STARLAM® 9:  200mm  240mm  300mm STARLAM® 15:  200mm  240mm  300mm Traverse colour:  White  RAL 9006  Black Other colours on request Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. rm | L-shaped perimeter trims Height: 40mm Width: 20mm Length: 2000mm Material: Aluminium, pre-coated Colour:  White  RAL 9006  Black Unit price: ............… €/rm Total price: ............… € |