# TICELL-N® | Thickness 0.6 mm | Single Blade Louvre – mounted on substructure U94

The Ticell-N® Single Blade Louvre is made up of a concealed grid, [U94 U-shaped carrier / V94 U-profile connector / C94 clips] and individually removable hingeable panels.

Ticell-N® panels are made up of cells by assembling and crimping perforated blades [square holes aligned in straight rows]. The blades have a woven appearance thanks to an exclusive durlumen® fabrication process.

Each blades has N-shaped cutouts along its length [in both directions of the panel, from bottom to top].

The blades are mounted in staggered rows between each N cut-out, giving the ceiling a multidirectional appearance in 4 dimensions.

Ticell-N® elements are mounted on U94 positioned at regular centers and generally corresponding to the length of the panel. The dimension of the center distance will be different when the panels are installed on the perimeter or in the corridors. The U94 is indexed to the pitch of the cells [sides] and the panels are fitted using 4 concealed C94 clips.

To increase the rigidity of the structure and maintain the centers of the U94, the use of a L-shaped primary carrier U 1040 (black lacquered) is recommended.

Panels and U94 U-shaped carriers are made of prepainted or anodized aluminum.

The elements making up the ceiling are manufactured in accordance with standard EN 13964 and the durlum standard. Regarding the suspension and fixing accessories, only composents authorized by the manufacturer must be used.

| m² | Panels – Single Blade Louvre Ticell-N® Aluminum thickness: 0,6mm Panel dimension [L x W]: 1211.04x605.52 mm Cell dimensions: 33,64x33,64 mm Height: 40 mm Perforation dimensions: ø3.5mm Color:  White  Black  Anodized Mill Finish  Other RAL........ Unit price : ............… €/m² Total price : .......... € |  |  |
| --- | --- | --- | --- |
| . |  |  |  |
| lm | U94 U-shaped carrier Color :  White  Black Length : 2400 mm Requirement per m²: 0,9 lm [panels 1200x600mm] Unit price : ............… €/lm Total price : .......... € |  |  |
| pce | V94 U-profile connector Color :  White  Black Requirement per m²: 0,40 units Unit price : ............… €/pce Total price : .......... € | …………………… pce | C94 clips Requirement per m²: 4 pcs/panel Unit price : ............… €/units Total price : .......... € |
| lm | L-shaped primary carrier U 1040 U 1040 Color : Black Length : 4000 mm Requirement per m²: 0.8 lm [panels 1211.04x605.52 mm] Unit price : ............… €/lm Total price : .......... € | …………………… pce | Connector U1041 Requirement per m²: 0.2 units Color : Black Unit price : ............… €/units Total price : .......... € |