# TICELL® | Single blade louvre – mounted on substructure U 94

Single blade louvre consisting of a substructure [U-shaped carrier] and prefabricated panels.

The specially perforated U-shaped carrier U 94 is suspended rigidly from the mounting level [e.g. bare ceiling] by means of M6 threaded rods using technical approved dowels. The distance between the U-shaped carriers depends on the specified module dimension.

The installation can be done by using the L-shaped primary carrier U 1040 for cross bracing if the suspension height will allow it. The cross bracing is required staring from a suspension height of 500mm. The L-shaped primary carrier is suspended rigidly and secured against loosening from the mounting level [e.g. bare ceiling] by means of M6 threaded rods using technical approved dowels. Afterwards, the U-shaped carrier U 94 is screwed to the L-shaped primary carrier.

The distance between the U-shaped carriers depends on the specified module dimension – according to the panel size length. The blades of the TICELL® single blade louvre have woven structure so that the use of a lighting from the ceiling void results in desired reflections or lighting effects. The panel elements are mounted at the module spacing and consist of square cells. The blades are perforated for connecting the panel elements and the U-shaped carriers by means of spring panel hangers [C 94]. The individual panel elements are self-adjusting and can be individually hinged open. A non-interchangeable marking [punched holes] ensures the correct alignment of the panel elements. The gearing of the panel elements ensures the joint of the blades in longitudinal direction.

Typically, the edge connection is carried out by means of a shadow gap.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – TICELL® single blade open ceiling Panel size [LxW]:  600x600mm  1200x600mm Material thickness: 0.6mm Cell dimension:  33.33x33.33mm  54.55x54.55mm Height: 40mm Colour:  White  Black  Natural anodized Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. rm | U-profile [U 94] Colour:  White  Black Length: 2400mm Requirement per m²: 0.83rm/m² [for panel 1200x600mm] Unit price: ............… €/rm Total price: ............… € |
| Pos. pcs | U-profile connector [V 94] Colour:  White  Black Requirement: 0.41 pc/m² Unit price: ............… €/pc Total price: ............… € |
| Pos. pcs | Retaining springs [C 94] Requirement: 4 pieces/panel Unit price: …............ €/pc Total price: …............ € |