# TOMEO-R® |  Ceiling light system

TOMEO-R® is a ceiling light combination consisting of round ceiling elements and round LUMEO®-R illuminated surfaces. Thanks to its different element sizes, TOMEO-R® is suitable for any room [possible diameters approx 300, 600, 900 or 1200 mm].

The elements can either be suspended using cables or installed directly on the raw ceiling or false ceiling using an M6 threaded rod with pipe sleeve. The system can be installed without the need for wall mountings, but can also be fitted flush to the wall with semicircular ceiling elements.

TOMEO-R® can also be mounted in connection with metal open cell ceiling [double-blade louvre type GEO®]. Perforated metal tiles [RG-L18] are hung in the suspended areas to make installation easier. Thanks to the modular character, the circular elements can be installed in any open cell. This guarantees high flexibility of the circular elements arrangement while also covering the ceiling void.

Technical details

Ceiling elements:	Round TOMEO ceiling elements made of perforated [RD-L30] or non-perforated metal in diameters of approx. 300, 600, 900, 1200 mm.

Light elements:	The LUMEO®-R elements [surface-mounted luminaire] are distinguished by homogeneous illumination. The luminaire cover consists of DUROSATIN®. In contrast to acrylic resin or glass, this material looks like a very high-quality material.

Tolerances and quality requirements according to TAIM, EN 13964 and durlum standard.

Acoustic requirements

The TOMEO-R® ceiling light system can be equipped with various absorption materials, for example with fleece-clad mineral wool. TOMEO-R® ceiling elements can also be supplied in perforated form with acoustic fleece-backed material. The RD-L30 perforation pattern is highly suited to this system.

Sprinklers

Sprinklers can be incorporated both into ceiling elements and into free areas between the elements. Depending on diameter and arrangement, only 1 sprinkler level may be necessary.

Ventilation

Ventilation can be integrated both into the round ceiling elements, for example as displacement diffuser, and between the elements as induction diffusers.

Cooling

TOMEO-R® can be installed as a cooling ceiling with copper meanders on the back pressed into aluminium profiles. As TOMEO-R® is an open ceiling light system, a high level of cooling can be achieved.

Leading brand durlum system TOMEO-R® ceiling light system or equivalent. Consisting of:

| Item pcs. | Delivery and installation of round ceiling elements [TOMEO-R® 300] Diameter: 293 mm Material: Electrolytically galvanised steel; t = 1 mm With welded-on frame and crossbar on the back Colour:  similar to RAL 9010 or alternatively  similar to RAL ….. Perforation:  smooth or alternatively  perforated [RD-L30] with fleece Optionally:  Designed as cooling ceiling Price per unit: ….. € Total price: ….. € |
| --- | --- |
| Item pcs. | Delivery and installation of round ceiling elements [TOMEO-R® 600] Diameter: 593 mm Material: Electrolytically galvanised steel; t = 1 mm With welded-on frame and crossbar on the back Colour:  similar to RAL 9010 or alternatively  similar to RAL ….. Perforation:  smooth or alternatively  perforated [RD-L30] with fleece Optionally:  Designed as cooling ceiling Price per unit: ….. € Total price: ….. € |
| Item pcs. | Delivery and installation of round ceiling elements [TOMEO-R® 900] Diameter: 892 mm Material: Electrolytically galvanised steel; t = 1 mm With welded-on frame and crossbar on the back Colour:  similar to RAL 9010 or alternatively  similar to RAL ….. Perforation:  smooth or alternatively  perforated [RD-L30] with fleece Optionally:  Designed as cooling ceiling Price per unit: ….. € Total price: ….. € |
| Item pcs. | Delivery and installation of round ceiling elements [TOMEO-R® 1200] Diameter: 1200 mm Material: Electrolytically galvanised steel; t = 1 mm With welded-on frame and crossbar on the back Colour:  similar to RAL 9010 or alternatively  similar to RAL ….. Perforation:  smooth or alternatively  perforated [RD-L30] with fleece Optionally:  Designed as cooling ceiling Price per unit: ….. € Total price: ….. € |
| Item pcs. | Delivery and installation of durlum LUMEO®-R 300 surface-mounted luminaire Article number: 15-LUM-R0300-002 LUMEO®-R surface-mounted luminaire round, direct beam. Aluminium body with frameless illuminated surface made of translucent DUROSATIN®, placed on an aluminium support ring. Cover and body are connected by means of a spring clamp system. Internal ballast unit for 1 22 W lamp T5-R. Protection type IP 20. Brand: durlum Product: LUMEO®-R surface-mounted luminaire Light distribution: direct beam Diameter: 293 mm Height: 82 mm Fitted with: 1 x 22 Watt T5-R Control:  non-dimmable or optionally  1-10 V dimmable or  DALI Protection class: IP20 Connection: 3-pin [5-pin] plug-in terminal or alternatively  version with Wieland plug and connecting cable Surface finish:  anodised aluminium or  similar to RAL ..... Cover: translucent DUROSATIN® on aluminium support Price per unit: ….. € Total price: ….. € |
| Item pcs. | Delivery and installation of durlum LUMEO®-R 600 surface-mounted luminaire Article number: 15-LUM-R0600-002 LUMEO®-R surface-mounted luminaire round, direct beam. Aluminium body with frameless illuminated surface made of translucent DUROSATIN®, placed on an aluminium support ring. Cover and body are connected by means of a spring clamp system. Internal ballast unit for 1 22 W lamp T5-R + 1 40 W lamp T5-R. Protection type IP 20. Brand: durlum Product: LUMEO®-R surface-mounted luminaire Light distribution: direct beam Diameter: 593 mm Height: 100 mm Fitted with: 1 x 22 Watt T5-R + 1 x 40 Watt T5-R Control:  non-dimmable or optionally  1-10 V dimmable or  DALI Protection class: IP20 Connection: 3-pin [5-pin] plug-in terminal or alternatively  version with Wieland plug and connecting cable Surface finish:  Anodised aluminium or optionally  similar to RAL ….. Cover: translucent DUROSATIN® on aluminium support Price per unit: ….. € Total price: ….. € |
| Item pcs. | Delivery and installation of durlum LUMEO®-R 900 surface-mounted luminaire Article number: 15-LUM-R0900-002 LUMEO®-R surface-mounted luminaire round, direct beam. Aluminium body with frameless illuminated surface made of translucent DUROSATIN®, placed on an aluminium support ring. Cover and body are connected by means of a spring clamp system. Internal ballast unit for 4 40 W lamp T5-R. Protection type IP 20. Brand: durlum Product: LUMEO®-R surface-mounted luminaire Light distribution: direct beam Diameter: 893 mm Height: 140 mm Fitted with: 4 x 40 Watt T5 Control:  non-dimmable or optionally  1-10 V dimmable or  DALI Connection: 3-pin [5-pin] plug-in terminal or alternatively  version with Wieland plug and connecting cable Colour:  silver or optionally  similar to RAL ….. Protection class: IP20 Cover: translucent DUROSATIN® and transparent DUROMASTER on aluminium support Price per unit: ….. € Total price: ….. € |
| Item pcs. | Delivery and installation of durlum LUMEO®-R 1200 surface-mounted luminaire Article number: 15-LUM-R1200-002 LUMEO®-R surface-mounted luminaire round, direct beam. Body made of steel sheet. Jacket cover as aluminium support ring, translucent DUROSATIN® applied to bottom side, clear film [DUROMASTER] applied to top side as dirt repellent. Cover and body are connected by means of a rotating mechanism. Internal electronic ballast unit for 4 x 40 W T5-R lamps. Protection class IP20. Brand: durlum Product: LUMEO®-R surface-mounted luminaire Light distribution: direct beam Diameter: 1200 mm Height: 200 mm Fitted with: 1 x 22 W + 5 x 40 W T5-R Control:  non-dimmable or optionally  1-10 V dimmable or  DALI Connection: 3-pin [5-pin] plug-in terminal or alternatively  version with Wieland plug and connecting cable Protection class: IP20 Colour:  silver or optionally  similar to RAL ….. Cover: translucent DUROSATIN® and transparent DUROMASTER on aluminium support Price per unit: ….. € Total price: ….. € |
| Item pcs. | 00-2855341 Pull-off tool for LUMEO® film element Price per unit: ….. € Total price: ….. € |
| Item m² | Optionally: Delivery and installation of durlum double-blade louvre type GEO® For the text for invitation to tender, please go to www.durlum.de. Price per unit: ….. €/m² Total price: ….. € |
| Item pcs. | Optionally: Delivery and installation of durlum perforated metal tiles [RG-L18] Price per unit: ….. € Total price: ….. € |