# VARIO 3® | Linear louvre – mounted on T15/T24 carrier

Linear blade open cell ceiling consisting of a substructure [T-shaped rail] and prefabricated panel elements.

The T-shaped rail is suspended rigidly from the mounting level [e.g. bare ceiling] via a suspension hanger for threaded rod with M6 threaded rods using technical approved dowels [distance of the suspensions according to EN 13964]. The distance between the T-profiles depends on the specified module dimension – according to the panel size length.

The panels are mounted at the module spacing and consist of cells with U-shaped, sharp-edged blades running in parallel. The VARIO 3® ceilings are composed of fine blades and two larger perimeter blades positioned circumferentially around the panel. The blades are connected to a panel via transverses. The individual panels are self-adjusting.

All parts are made of pre-coated aluminium.

| Pos. m² | Panels – VARIO 3® linear blade open cell ceiling Panels:  600x600mm  1200x600mm Blade: 3/20mm [blade width/blade height] Blade distance/module: ............... mm [15-40mm] Blade colour:  White  RAL 9006  Black Perimeter blade:  4/42mm [blade width/blade height]  9/40mm [blade width/blade height]  15/37mm [blade width/blade height] Colour of perimeter blade:  White  RAL 9006  Black  Natural mirror Traverse: 9/40mm [blade width/blade height] Traverse distance: approx. 300mm Traverse colour: Black Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. m² | T-shaped rail [T15/T24] Colour:  Black Other colours on request Unit price: ............… €/m² Total price: ............… € |