# VARIO 3® | Linear louvre – mounted on substructure U 94

Linear blade open cell ceiling consisting of a substructure [U-shaped carrier] and prefabricated panels.

The specially perforated U-shaped carrier U 94 is suspended rigidly from the mounting level [e.g. bare ceiling] by means of M6 threaded rods using technical approved dowels. The distance between the U-shaped carriers depends on the specified module dimension.

The installation can be done by using the L-shaped primary carrier U 1040 for cross bracing if the suspension height will allow it. The cross bracing is required staring from a suspension height of 500mm. The L-shaped primary carrier is suspended rigidly and secured against loosening from the mounting level [e.g. bare ceiling] by means of M6 threaded rods using technical approved dowels. Afterwards, the U-shaped carrier U 94 is screwed to the L-shaped primary carrier.

The panels are mounted at the module spacing and consist of cells with U-shaped, sharp-edged blades running in parallel. The VARIO 3® ceilings are composed of fine blades and three larger intermediate blades. The blades are perforated for connecting the panels and the U-shaped carriers by means of spring panel hangers [C 94 clips]. The blades are connected to a panel via transverses. The individual panels are self-adjusting and can be individually hinged open. A non-interchangeable marking [punched holes] ensures the correct alignment of the panels.

All parts are made of pre-coated aluminium.

| Pos. m² | Panels – VARIO 3® linear blade open cell ceiling Panels:  1200x600mm Blade: 3/20mm [blade width/blade height] Blade distance/module: ............... mm [15-40mm] Blade colour:  White  RAL 9006  Black Perimeter blade:  4/42mm [blade width/blade height]  9/40mm [blade width/blade height]  15/37mm [blade width/blade height] Colour of perimeter blade:  White  RAL 9006  Black  Natural mirror Traverse: 9/40mm [blade width/blade height] Traverse distance: approx. 300mm Traverse colour: Black Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. rm | U-profile [U 94] Colour:  White  Black Length: 2400mm Requirement per m²: 0.83rm/m² [for panel 1200x600mm] Unit price: ............… €/rm Total price: ............… € |
| Pos. pcs | U-profile connector [V 94] Colour:  White  Black Requirement: 0.41 pc/m² Unit price: ............… €/pc Total price: ............… € |
| Pos. pcs | Retaining springs [C 94] Requirement: 4 pieces/panel Unit price: …............ €/Stk Total price: …............ € |