# VARIO® 3 | Linear louvre – mounted with integrated carrier

Linear blade louvre consisting of a substructure with integrated carrier and prefabricated panel elements.

The integrated carrier is suspended rigidly from the mounting level [e.g. bare ceiling] with suspension hangers and M6 threaded rods using technical approved dowels. The distance between the integrated carriers depends on the specified module dimension.

The panels are mounted at the module spacing and consist of cells with U-shaped, sharp-edged blades running in parallel. The VARIO 3® ceilings are composed of fine blades and four larger perimeter blades [circumferential]. At the crossing point, both the perimeter blades and the integrated carriers are continuous and create a uniform picture. The blades are connected to a panel via transverses. The individual panels are self-adjusting.

All parts are made of pre-coated aluminium.

| Pos. m² | Panels – VARIO 3® linear blade open cell ceiling Panels:  600x600mm  1200x600mm Blade: 3/20mm [blade width/blade height] Blade distance/module: ............... mm [15-40mm] Blade colour:  White  RAL 9006  Black Perimeter blade: ( 4/42mm [blade width/blade height]  9/40mm [blade width/blade height]  15/37mm [blade width/blade height] Colour of perimeter blade:  White ( RAL 9006  Black  Natural mirror Traverse: 9/40mm [blade width/blade height] Traverse distance: approx. 300mm Traverse colour: Black Material: Aluminium Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. rm | Integrated carrier Colour:  White  RAL 9006  Black  Aluminium natural mirror Other colours on request Length: 2400mm Incl. 1 connector per carrier Incl. 2 suspension hangers per carrier Unit price: ............… €/rm Total price: ............… € |