# dur-SONIC® CUBE broadband absorber

|  | Delivery and installation of dur-SONIC® CUBE broadband absorbers in the ceiling void. To improve acoustics and speech intelligibility, individual absorbers are arranged in the ceiling void and attached either directly to the substructure or in the ceiling void using the corresponding 45-DSC-HALTER-01 fastening brackets. The square-shaped absorbers have flow-optimised acoustic filling. Rated sound absorption coefficients Alpha-w according to ISO 11654 depending on density of the used absorbers must be verified by test certificates from an independent testing institute: dur-SONIC® CUBE with a length of 800 mm, 2 pieces/m² ceiling area: Alpha-w 0.80 Material: Galvanised steel Dimensions: 160 mm x 160 mm / square Filling: Flow-optimised acoustic filling Perforation: L30 - Rd 1.5-2.83 / 22 % Surface finish: powder-coated Colour: RAL 9011 matt (standard) Fastening: 2 fastening brackets per element (supplied loose) Fire protection classification: A2-s1, d0 according to DIN EN 13501-01 |
| --- | --- |
| Pos ............................ ............................ pcs. | Single length: 400 mm |
| Pos ............................ ............................ pcs. | Single length: 600 mm |
| Pos ............................ ............................ pcs. | Single length: 800 mm |
| Pos ............................ ............................ pcs. | Single length: 1000 mm |
| Pos ............................ ............................ pcs. | Start-up costs depending on format and call-up |