dur-SONIC TUBE | Absorber

Absorber consisting of perforated metal cylinders [Standard perforation with free cross-sectional area 25%] backed with acoustic fleece [building material class: A2-s1, d0 according to DIN EN 13501-1] and filled with mineral wool [building material class: A2 according to DIN 4102]. With end covers on both sides and threaded rivet nuts to fasten hangers. No release of free fibres due to completely sealed casing. Surface is easy to clean.

Standard lengths:	400mm, 600mm, 800 mm

Diameter:		200 mm

Surface:		Powder-coated, [colour black]

To be mounted in ceiling void either hanging from raw ceiling or attached to substructure of open cell ceiling underneath depending on density of installations therein. Installation direction either horizontal or vertical, depending on acoustic requirements and space available.

Acoustic requirements:

For different spacings (centre axis tube - centre axis tube) following absorption values Alpha w according to DIN EN ISO 354 are to be verified by an independent test institute:

Spacing:	1000mm	Alpha w: 0,60

Spacing:	 600mm	Alpha w: 0,80

Spacing :	 400mm	Alpha w: 1,00

Lead product: durlum system dur-SONIC TUBE or equivalent consisting of:

| Pos.............pcs | Supply and mount dur-SONIC TUBE absorbers Diameter: 200mm Length: .............mm Surface: powder-coated, black Installation direction (horizontal / vertical) to be determined including hangers unit price: ..............€ / pc total price: ..............€ |
| --- | --- |
| Pos.............pcs | set-up cost for each format, design and call-off |